HEAT PRESS MACHINE

Operator’s Manual

This machine is not suitable for minors to use or food processing.

• General:

A heat press machine is the machine that presses a transfer onto an imprintable substrate. Using high temperatures and pressures for a certain amount of time, the transfer is permanently embedded into the product.

Heat presses are recommended for professional and satisfying results simply because standard laminating devices and home hand irons can not get even near the temperatures required for a reliable transfer.

•Components and functions:

power switch

digital controller

pressure knob

④handle

⑤heating plate

1.Set temperature

| Press OK button | The display show P-1 | Press arrows “△”or “▽” adjust temp you need |
| --- | --- | --- |

- Set time

| Press OK button after setting temp,display show P-2 | Press arrows “△”or “▽” adjust time you need | Press OK button after time setting |
| --- | --- | --- |

- Place the material on working table,adjust the pressure knob(3) get right pressure required

- Connect power plug (1) and turn on power

- Set time & temp you want with temp/time gauge (2) get required numbers.

- The temp for heating plate (5) will start to rise up,when it reaching temp you set,place the material on the working table and close the handle (4).

- The buzzer will sound when reaching time you set,release the handle(4) and take out the material

Note: 1.Because the materials and thickness of these materials are different,so the temp and time also    required different.

2. Pls try a small sample to get the right temp and time needed before large quantities printing.

3.Clothing require temp about 180℃,Fabric clothing use with sublimation paper:30-50 sec,100% cotton

clothing use with transfer paper:15-20sec.

|  | Problem | Cause | Handling methods |
| --- | --- | --- | --- |
| 1 | Picture burning | Too much time or temp | Adjust time or temp. |
| 2 | Fails to heat up,but display well | Heating plate or Relay broken | Check the heating plate or Relay |
| 3 | Lack of color, faded image. | Insufficient temperature and/or press time. | Adjust the temperature and/or increase the heating time. |
| 4 | Darkened, deep color; blurred image. | Pressed for too long,temperature too high | Adjust the temperature and heating time. |
| 5 | No action after turn on power | Plug whether touch well Fuse whether burn | Check the plug and fuse |
| 6 | The real temp pass over setting temp | Relay broken | Replace the relay |