Installation manual for plotter  driver

This installation package is just a driver installation file and cannot be output directly as a printer.

一． installation steps

- Start the plotter, set the plotter to print mode ("Printer Y"), and connect the plotter to a computer using a USB cable;

- Select the correct USB installation port: Right-click "My Computer" (or "Computer") on the system desktop and click "Manage"

- Click "Device Manager" to find "Universal Serial Bus Controller", "USB Printing Support" (or "USB Printer Support") appears, right-click to view properties (if you can't view normally, please see "FAQ")

- Click "Details" to find "Bus Relationship" in "Device Description"

- Check the "Bus Relationship" value, and finally the "USB001" message appears, that is, when installing the printer driver, select the "USB001" port. (Keep in mind this port, it will be used for installation and data sending!)

- Select the appropriate installation program according to your computer system. How to select the "Win32.exe" installation file if your computer's operating system is 32-bit, and select the "Win64.exe" installation file if your computer operating system is 64-bit;

- Select “USB001” as the installation port (according to step 2) and click “Install”

- At this time, you can click “Test”, and the lettering machine will engraved a “10 × 10mm” rectangle; (this step can be ignored);

- Uninstall the printer driver. Double-click the installation file and click "Delete" to uninstall the driver.

- Software output test (Take Artcut software as an example)

- Open "Artcut" software

- Click "cut Output"

- Select "USB005" port (see installation step 2)

- Click "cut Output"

- Click "Start"

- other software same as artcut

- 三.FAQ

- Can't see "USB Printing Support" (or "USB Printer Support") or "Exclamation mark" in Device Manager?

- 1）If you cannot see "USB Printing Support" (or "USB Printer Support"), please check if the plotter is set to "Printer Y" and if there is a problem with the USB cable connection;

2）If an exclamation mark appears in front of "Print Support" or "USB Printer Support", it means that the printer driver does not come with usbprint.sys file in your computer system.

The usbprint.sys file has the same file name but different file contents on different computer systems.

At this time, please download and copy the usbprint.sys file to your computer, right-click the universal serial bus controller in the device manager, and click "Update Driver". After the driver update is successful, the exclamation mark should disappear.You can observe the correct usbprint.sys file installation directory by right-clicking the Universal Serial Bus controller in the Device Manager and clicking "Properties".

- Identify the computer operating system?

- 1）Press the shortcut key Win + R to open the running window.

- 2） Enter the following command in the run window: dxdiag

- 3）After running the above command, the following interface will pop up, and you can see the specific information of the system below:

- 4）The operating system is Windows 7 Ultimate. The system digits are 64 bits.

- Note: For Windows XP, if there are not many digits displayed, it is 32 as the operating system.

- "Unreachable" or "Add Printer error" during installation?

- Right-click the installation file and run as administrator.